Theoretical study of thermogalvanic cells in steady state.

Artjom V. Sokirko* ,

Dep. of Hydromechanics, The Royal Institute of Technology,

10044 Stockholm, Sweden

Abstract

The functioning of thermogalvanic cells has been studied theoretically. The analysis for the simplest case with the account for only diffusion transport of the reacting substances from one electrode to another has been made, the simple relation for thermodinamic efficiency of such energy sources was obtained. The diffusion-migration transport for the horizontally arranged cell and the diffusion-convection transport for the vertically arranged cell were studied in detail. The original mathematic methods for solving of transport equations were used. The analytical results for the concentration distribution and the volt-amper curves were calculated. The principal role of the electrode kinetics (fast changing of the equilibrium electrode potential with the temperature) was shown. At the end the general algorithm for improving of thermogalvanic cells is proposed.

Keywords: diffusion-migration transport, fluid motion, efficiency, concentration distribution, volt-amper curve.

Introductions.

Direct transformation of the thermal energy into the electrical one by means of thermogalvanic cells (TGC) on the electrochemical basis is one of the most attractive way of the energetics development. The characteristic features of those systems, such as absence of any moving mechanical particles, absolutely complete and closed usage of solutions, longevity and simplicity in use are their obvious merits [1-4].

Such kind of research is carried out most actively in Japan due to significant amounts of geothermal water supplies. That research includes experimental study of functioning of different TGC variants, as well as model calculations by supercomputers [5-6]. The main shortcoming of TGC is their extremely low efficiency, that makes impossible their application in common practice. The main hope of researchers of TGC is the possibility to find experimentally the characteristics of the device (the temperature of electrodes, contents, geometrical size), which would provide for the highest efficiency. Model calculations of such systems demand significant computer time. It prevents from systematic research of all possible variants of TGC. Actually, only a few calculations for some variants of TGC, more or less related to those real systems that already exist, were made [7-8]. That is why it was necessary to work out a reasonably simple theoretical model which could help in study of the principal properties of TGC and also to provide for theoretical estimation of the present TGC constructions efficiency.

The first three parts of the present paper are devoted to general properties of TGC and also to the qualitative estimation of their efficiency. The main steps of obtaining the volt-amper curves for TGC are described in the fourth part. It allows us to obtain more accurate evaluation of the efficiency. The last two parts are devoted to discussion of the fluid flow in the volume of TGC and to arising distribution of concentrations under the diffusion and convection effects.

1. TGC as an electrical system.

TGC is the usual source of current (Fig.1). The effective voltage, as well as the electric current in the system, varies with of the external resistance Rext change. The qualitative behaviour of I(Rext) and U(Rext ) dependencies is shown in Fig.2. When Rext Æ0, the current I  tends to I0 - the current of short circuit, and when Rext Æ_  and I Æ0,  the voltage on the load tends to U0, the voltage on terminals at open circuit. In the general case the inner resistance r  can be calculated from the second Kirchhoff relationship for the closed circuit:             r =U0/I- Rext ; r is a function of Rext, as well as useful power W=UI, educed on Rext  (Fig.2). But in the main range of Rext  the function r(Rext) is almost constant, equal to inner resistance r0  of the open circuit.

It can be easily seen that power Wmax, educed on the external resistance, becomes maximal under the condition of equality of inner and external loads r0 _Rext  and is equal to

Wmax _F(u02r0 ;4) 				       (1)

The same power, obviously, is educed on the inner resistance r0.

The most important difference of TGC from usual electrochemical sources of current is that the useful capacity is produced at the expense of temperature difference between electrodes and does not depend on power, educed on the inner resistance. Even in the case of the open circuit, almost the same energy is required for maintaining the temperature difference between electrodes, as for working in the optimum regime. That is why Wmax, given by formula (1), is the optimum value. Thus, to avoid the additional losses of power, the electrical resistance in the external device should be always constant and equal to inner resistance of TGC.

Some experimental volt-amper dependencies for TGC are given in [5]. They look typically for electrochemical systems, i.e. the current reaches limiting values in the case of enough high modules of potentials difference. The only difference is that this curve does not pass the zero point U=I=0. The point of intersection of this curve with the axis U (U=U0,I=0) relates to completely open circuit, the point of intersection with the axis I(U=0,I=I0) relates to the short circuit. These two points are situated not far from each other (in comparison with the characteristic value of the limiting current in the system) and get closer when the temperature difference reduces. Obviously, a certain regime of TGC functioning, i.e. a certain value of Rext, correlates to each point of the curve segment between U0  and I0. The points outside this segment principally can not be reached when the TGC is used as the current source. It means that the real TGC function in a regime, far from the limiting one (near the equilibrium), when the volt-amper curve can be considered approximately straight line, which does not pass the zero point.

2. Dimensional analysis of TGC functioning as thermodynamic system.

Let us consider the simplest TGC model, consisting of two large parallel plates placed in a small (in comparison with their size) distance L  from each other and heated up to different temperature values T1 and T2. Then the thermal density flux jh  from the heated plate to the cold one can be calculated as

jh = æh F(T2 -T1; L),					(2)

where æh  is the coefficient of thermal conductivity of liquid between the plates. In (2) we presumed that there was no convective flow of solution. Sometimes instead of the thermal conductivity coefficient æh  it is more convenient to use the temperature conductivity coefficient: æT = æh /O(c;^)r , where r is the density of liquid and O(c;^)  is specific thermal capacity.

The maximum density of electric power je  can be calculated as je =Wmax/s where s  is the area of plates. The inner resistance of TGC r0  can be presented as r0 = æi L/s where æi is the specific resistance and can be evaluated as        æi =c0l0, where c0  being the concentration of solution, l0 is the equivalent electroconductivity. Thus, the electric power density can be presented as

je = F(u02c0l0;4L).				    (3)

The efficiency of thermodynamical engine can not exceed  hCarno   of the ideal thermodynamical system, functioning by Carno cycle:

hCarno = F(T2 -T1;T2).				       	(4)

That is why it is more convenient to describe the efficiency h of real engines by the ratio of their efficiency to the efficiency of Carno cycle hCarno . The introduced efficiency h  lies within the limits 0<h <1:

h = F(je;jh)  F(1;hCarno  ) = B(F(u0;T2 -T1))SUP15(2 )F(T2c0l0;4æh ).			(5)

The only dependent parameter u0, involved in the equation (5), can be obtained from the analysis of TGC electrochemical kinetics.

3. Electrochemical model of TGC functioning.

The real TGC is an electrochemical system with concentrated mixture of two salts, for example,  FeCl2 and  FeCl3, placed between two identical unconsumed electrodes. The same electrochemical reaction takes place on the both electrodes:

Mz1++ (z1-z2)e- O(k1;Æ;¨;k2) Mz2+.				(6)

Rate constants of forward (k1) and back (k2) reactions depend on the temperature and that is why they are different for different electrodes. Equilibrium concentrations, defined by ratio k1/k2, appear different at different electrodes. It causes ion transport from one electrode towards another, and provides for the reaction process in anodal direction on one electrode, and in cathodal direction on the other one.

In accordance with generally accepted designations [9], density of electric current, flowing through the surface of the electrode, can be presented as follows [10]:

i=i0 B(F(c1|s;c10) expB(F(-(z1-z2)(1-a)Fx;RT) ) - F(c2|s;c20) expB(F((z1-z2)aFx;RT)));  	(7)

where i0 is the exchange current density, c1|S and c2|S are concentrations of ions Mz1+ and Mz2+ in the vicinity of the electrode, cO(1,0) and cO(2,0) are the corresponding initial concentrations, T is the absolute temperature in the vicinity of the electrode, R is the universal gas constant, a  is the symmetry factor, x  is the surface overpotential:

x  =j  -  j (s)					 (8)

j  is the potential drop between the nearest to the electrode zone and the electrode, j (s) is the equilibrium potential.

The most frequently used systems for studying of TGC are Fe(CN)3-6 | Fe(CN)4-6 (standard potential j (s)= 0,36V at T=250C) and Fe2+| Fe3+ (standard potential j (s)= 0,771V at T=250C). But in the definition (8) the equilibrium potential, measured at the real temperature of the electrode, is included instead of standard potential, measured at standard temperature

For the open circuit the electrical current i equals to zero, therefore the potential drop inside the solution also equals to zero. It means that the potential drop between electrodes may be calculated just as u0  =   j (s)| T=T2 -  -j(s)| T=T1, where T1 and T2  are the electrodes temperature values. Introducing u0  into (5), we obtain

h  = B(F(j(s)|T=T2 -j(s)|T=T1  ; T2 -T1 ))SUP18(2) F(T2c0l0;4æh )

or in the case of small temperature difference between the electrodes (T2 -T1 ) «T2

h  = B(F(dj(s);dT ))SUP18(2) B(F(Tc0l0;4)) B(F(1;æT O(c ;^)r )).			(9)

In expression (9) we assumed that the standard potential of the electrochemical reaction depends on the temperature only. Introducing the parameter value into (9) in case when the concentrations of FeCl2 and FeCl3 in the mixture are both 1.0 M, we get the approximate estimation for h ~10-4, which agrees well with the experimental data [5].

As follows from relation (9), the efficiency of TGC is defined as a product of three terms. The first of them characterises the type of the electrochemical reaction. In order to increase h, the electrochemical reactions that have the strongest dependence upon the temperature, should be used, and the temperature range where |dj(s)/dT | reaches its maximum should be chosen.

The second term mainly characterises the electric conductivity of a cell. As it was mentioned by the majority of researches, solutions should be concentrated up to the maximum value. In this case parameter l0 is not independent, but is completely defined by the composition of solution, concentration and its temperature. It is important to emphasize, that in the case under analysis electroconductivity of the solution can not be increased by means of addition of any supporting electrolyte, because the electric current in the system is conducted only by ions Mz1+ and Mz2+. The average value of the absolute temperature T  in the second term of (9) is varying within narrow limits and can not significantly change the efficiency h ; moreover, as it was already mentioned above, T  is to be chosen deliberately in order to maximise the first term.

The last term is inverted thermal conductivity of the solution, i.e. of the solvent. Its value is, actually, constant for all water solutions and only slightly depends on the temperature.

Beside, the analysis of the factors, upon which the efficiency h  does not depend, is not less important. First, the efficiency h  does not depend on the temperature difference between the electrodes (if it exists), because the potential difference u0  in (5)  is proportional to (T2 -T1 ) with high accuracy. Second, the efficiency h  does not depend on any geometrical sizes of the system, in particular, on the distance between electrodes. Geometrical sizes influence on the appearance of the natural convection in the liquid. Convection promotes the transport of electroactive ions from one electrode to another and reduces electric resistance of the system. But, at the same time, convection increases the heat transport from one electrode to another, and the processes of mass and energy transport increase to approximately the same extend. It leads to the independence of h  of convection (natural or artificial). In detail the phenomenon of convection in TGC is analyzed in parts 5-6.

4. Diffusion-migration currents in TGC.

Let us consider TGC in the absence of convective flows. It can be either horizontal cell with heated upper electrode or the one with heated lower electrode (if the hydrodynamical Ralay number  Ra  is lower then the critical one, when the convective flow, as in Bernar cells, arises). Convection can also be suppressed by injection of a porous filler between the electrodes or of some other separators [11].

The distance between the electrodes is much less than their size, consequently, it may be assumed that the processes are described by one-dimensional transport equations. The temperature distribution in this case is calculated trivially: it changes linearly from values T1 on one electrode to T2  on the other. The distributions of the cation concentrations c1 and c2  with the charge number z1 and z2 respectively, and anions concentration c3  with the charge number z3  are determined by the system of Nernst-Plank equations:

i1 = z1 F(D1  F(dc1;dx) + u1 c1  F(dj;dx));				(10)

i2 = z2 F(D2  F(dc2;dx) + u2 c2  F(dj;dx)),				(11)

0 =     D3  F(dc3;dx) - u3 c3  F(dj;dx) ,				(13)

z1 c1  + z2 c2 = z3 c3 .					(14)

Here Dk,uk  are diffusion coefficients and mobility of the k-th ion (k=1,2,3), accordingly, i1  and i2  are current densities, transported by ion species 1 and 2, F  is the Faraday number, j   is the electric potential, x  is the coordinate, directed from the left electrode (x = xL) towards the right one (x = xR ), xL £x £xR. Relation (13) expresses the condition of electroneutrality.

It follows from the stoichiometry of the electrochemical reaction, that

F(i1 ;z1 )  = - F(i2 ;z2)

and, consequently, the density of the integral electric current density can be presented as

i = i1  + i2  =i1  (1 - z2 /z1 )				(15)

The cathode reaction on one electrode occurs in stationary conditions with the same rate, as the anode reaction on the other electrode. That is why the total amount of substance of each sort remains constant and is defined by the initial concentrations only:

I		(xL;xR;Ck(x)dx) = (xR-xL)C0k					(16)

Beside, the two boundary conditions are valid on each  electrode:

i=i0B(F(c1|xL;cO(1;0)) expB(F(-(z1-z2)(1-a)FxL;RT1) ) - F(c2|xL;cO(2;0)) expB(F((z1-z2)FaxL;RT1))) ,  (17)

i=i0 B(- F(c1|xR;cO(1;0))expB(F(-(z1-z2)(1-a)FxR;RT2)) + F(c2|xR;cO(1;0)) expB(F((z1-z2)FaxR;RT2)))  . (18)

For simplicity, we will further assume a =1/2 . The potential drop u between two electrodes is algebraic sum of the potential drops on the electrolyte solution boundaries and inside the electrolyte.

u = -(j (s)|T=T2  +x R) + (j |x=xR  -j |x=xL ) + (j (s)|T=T1  +x L).      (19)

We assume, that the Einstein correlation for mobility is valid:

uk =Dk  F(Fzk ;RT )					(20)

that is, strictly speaking, true only for deluted solutions. Obviously, the mobilities and the diffusion coefficients also depend on the temperature. But we suggest, that the temperature difference is not too high, so that the mobilities and the diffusion coefficients can be considered constant in the whole volume.

It can be verified aposteriori, that the arising concentration difference is insignificant, therefore the mobilities and diffusion coefficients dependence upon the concentrations can be definitely neglected.

Expressions (10)-(20) are complex system of differential, algebraic and integral (16) equations. In order to find the volt-amper dependence U(I) from (10)-(20), it is necessary that the profiles of concentrations and potential should be determined. Differential equations (10)-(12) are nonlinear, because they include products of two unknown functions: concentration and potential. Boundary conditions are also nonlinear (with the exponential terms).

Luckily, this complex mathematical problem allows to get an exact analytical solution, that does not require any additional approximations or linearization. This solution in case of electrodes with equal temperature and the only electroactive species of ions was given in [12]. The principal steps of generalization of that solution for the case of different electrode temperatures and two electroactive ions, and also the final formulas for the volt-amper dependency, are given in Appendix A.

The main principle of the solution [12] is that instead of seeking the direct dependence I(U), the parametrical dependencies I(w) and U(w) should be determined. The dimensionless drop of potential in the transport zone j |x=xR  -j |x=xL  is used as an intermediate parameter w. With these expressions for I(w) and U(w) it is possible to plot the curve of for the volt-amper dependence, as well as by means of functions I(U) or U(I) .

The volt-amper curve obtained by theoretical calculations for one of systems Fe(CN)O(6 ;4+) | Fe(CN)O(6 ;3+), studied experimentally in detail in the number of papers [5-7], is given in Fig.3. In the same figure the volt-amper curve for the case, when the diffusion-migration transport is not the limiting stage, is dotted. In this case we can accept the approximation c1 _cO(1;0), c2 _cO(2;0), j _j (s)   for all values of the coordinate x.. Thus, the expressions (17)-(19) give the elementary expressions of the volt-amper dependence I(U) .

It is shown in Fig.3, that the both curves begin at the same point Uo  on the axis I, related to the open circuit. In the point Uo/2, corresponding to the optimum regime of TGC functioning, the curves can be considered not distinguishable. The noticeable divergencies appear near the region of the short circuit and further towards high values of voltage, that are not realized in practice. It happens because the exact and the approximate solutions have principally different asymptotics at high potential values. The exact solution tends asymptotically to the limiting current, but the approximate one increases unlimitedly (due to the constant concentrations near the electrode).

Another advantage of the exact solution given in the Appendix A is worth mentioning. For the fixed value of w, i.e. for the certain current density I and the potential difference U, it is possible to plot the profiles of concentrations ci(x) (see Fig.4). As well as the curve I(U), these curves are plotted in parametrical form of (ci(f), x(f) ); the value of the electric potential f  in the transport zone volume is used as an intermediate parameter. These profiles of concentrations are given for the above mentioned electrochemical system in the regime of the maximum efficiency, i.e. at U=U0/2. In the same figure the integral concentration c=c1+c2+c3 is given. One can see, that although the concentrations c1 and c2  vary, their changes compensate each other, so that the difference of the total concentration Dc  and concentration of anions Dc3  is not significant: Dc «c. It also proves our suggestion, that the coefficients of diffusion and mobility can be considered approximately independent of the concentrations (because concentrations themselves do not change significantly).

5. Convective flow of the liquid in TGC.

Let us consider an ordinary TGC with vertical electrodes, where the distance between the electrodes is much less than the cell size (fig.4). In such a cell the two-dimension flow of liquid with the velocity v, which can be described by the stationary Navier-Stokes equation with additional terms in Bousinesque form, arises:

- F(1;r) –p + n  D O(v;Æ) - bT g(T-T0 )O(e;Æ)y + bC g(c-c0 )O(e;Æ)y = 0.	       (21)

Here p  is pressure, n  is kinetic viscosity,  g is gravitation constant, bT  is the thermal expansion coefficient, bC is the coefficient of dependence of the density on concentration, O(e;Æ)y  is the ort in the direction of axis y, T0 , C0  and r  are the average values of temperature, concentration and density. The characteristic rate of the liquid motion |v| is extremely low: |v|~10-4 m/s, the characteristic size of the system (the distance between the electrodes) is ~ 10-3 m, n  for water is approximately equal to 10-6 m2/s. Therefore, the Reynolds number is Re~ 0.1, that gives the ground to neglect the nonlinear term in (21).

The pressure gradient term can be eliminated by taking the curl of Equations (21). In the terms of the stream function, which is correlated to v  by expression O(v;Æ) = ( F(_F;_y) ;-  F(_F;_x)), we determine that

n  D2F = bTg F(_T;_x) - bTg F(_C;_x) .			        (22)

Let us compare the typical values of two terms in the right part of (22). In accordance with the results of reference [7], if the convective flow of liquid takes place under the influence of the temperature difference of 50 K0, the arising difference of concentrations is 0,08 M. Introducing bT =3.10-4 m3K-1 and bc =5.10-2 m3mol-1 in accordance with the table data from [13-14], we get that the first term in the right part of (22) is four times as much as the second term. It means, that the convection flow of liquid takes place mainly due to the temperature gradient, but not due to the concentration gradient.

The other simplification is connected with the fact, that. in the main bulk volume of liquid the thermal flow density from one electrode to another is constant, that provides for the temperature profile remaining linear [7]. Some deviations from linearity are observed in the upper and lower parts of the cell, where the convective thermal transport becomes significant. It occurs, because there is the heat transport from one electrode to another in the main volume of liquid, i.e. in the horizontal direction, and the liquid flow is directed upwards or downwards, i.e. in the perpendicular direction and can not cause any significant convective heat transport. These reasons were also supported by computer modelling; its detailed description is beyond the scope of this paper.

Thus, in the middle part of the cell (excluding two approximately square upper and lower domains) the derivative _T/_x  can be replaced by the constant temperature gradient and we can present (22) in the following form:

D 2F = q						(23)

where

q = F(bTg (T2-T1 );n (xL-xR ))						(24)

is a certain constant. The equation of the viscouse liquid motion (23) is to be completed by boundary conditions in the absence of tangential rate component on the electrode surface:

F(_F;_x)|x=xL = 0;   F(_F;_x)|x=xR = 0				(25)

and also by the condition of the absence of the normal rate component. The latter is equivalent to the statement, that the boundary between the liquid and solid phases is one of  F  = const, and the constant can be chosen arbitrary:

F |x=xR  = 0 ;  F |x=xL = 0				(26)

In the main volume the liquid flow does not depend on boundary conditions on the upper and lower boundaries of the cell. Equation (23) with boundary conditions (25), (26) gives the elementary solution:

F  =  0.25 q (x-x L)2(x-x R)2				(27)

or, for the rate:

O(v;Æ) = q (x-x L)(x-x R)(x- F(xL+xR;2))O(e;Æ)y			(28)

The liquid is stagnant along the vertical middle line due to the axial symmetry of TGC. Expression (28) will be used further for solving the problem of diffusion-convective flows of cations under the influence of the forced convection. So, the general problem of diffusion-convective transport of mass and energy can be devided into three consequent steps: 1) calculating of the heat transport problem; 2) solving of the subproblem of the generation of natural convection of liquid; 3) solving of the subproblem of cations of two species transport under the influence of the diffusion and the given fixed convection.

Such a division is mainly stipulated by the physical sense of the problem. The existence of the temperature difference between the electrodes (the first-order effect) leads to two consequences (the  second-order effects): appearance of the heat convection in liquid and of the electric current flow. The latter causes a certain redistribution of concentrations, and it leads to the concentrational convection of liquid (the fourth-order effect).

Certainly, the quantitative evaluations are necessary in order to support these qualitative considerations, and that was done at the beginning of this part, but these considerations provide for better understanding of the reason why the heat convection should be taken into account (the second-order effect) and the concentrational convection should not (the fourth-order effect).

6.   Diffusion-convection currents in TGC with vertical electrode arrangement.

The diffusion coefficients of ions are much less than the heat conductivity coefficient, consequently, the process of ion transport and energy transport in TGC occur in two absolutely different way (see part 5).

In general, the cation transport of from one electrode to another can be divided into three stages:

1)  Diffusion transport of an ion from the electrode surface inside the moving liquid.

2)  Convection transport along the surface to its end; then, in the upper (or lower) part of the cell ions are transferred from one electrode to another by the liquid flow; then they keep moving along the surface of the other electrode.

3) Diffusion transport from the moving liquid towards the other electrode.

So, all the changes of concentration are located in the thin boundary layer near the electrodes and the initial values of all components remain constant in the main central volume.

We take into account the fact, that the cell height H is much more than its width L=(xR-xL), and introduce the dimensionless "surface" system of coordinate(n,t). The origin of this new coordinate system is the upper left corner of the cell, the normal axis O(n;Æ) is directed from the electrode into the solution, and the tangential axis O(t;Æ) consists of two components: the first one is directed along the surface of the first electrode downwards, and the second one is directed along the surface of the second electrode upwards (see Fig.5); the both components are directed along the direction of the liquid flow. There is a segment of the length L that is missing at the joint point of those two components. But there are no electrode processes in this region, and its length is much less than the length of the electrodes L«H; so we can expect that concentration and its derivative will be practically continuous in this point. The same considerations can be applied to the other joint point in the origin. The dimensionless equation of the convective diffusion for cations of the kind m is:

Dm F(_2cm;_n2) = F(_cm;_t) n B(F(qL2H2;2)) ,  m = 1,2			(29)

the value of H is chosen as the length scale. In the equation (29) the common approximation for transport in the boundary layer was done: in the expansion of the rate function v along the coordinate x only the linear term was left at x=xR  (or x=xR ), that relates to the absence of the lateral diffusion, as the convective transport is much more significant in that direction).

Functions c1, c2 are to be continuous in the points t=1 andt=2(t=0), i.e. to be periodical functions of the coordinate t  with the period 2. Beside, the solutions of the eq. (29) are to tend to the volume values cO(1;0) and cO(2;0) at a long distance from the electrode, i.e. when n has rather high value.

Although equations (29) look rather simple, they allow to describe pretty well all three stages of transport, listed at the beginning of this chapter. The main difficulty in their solving is that the boundary conditions inevitably include both surface concentrations c1|S andc2|S. That is why both equations (29) are to be solved simultaneously.

Migration transport is not accounted for in the problem (29), which means that the supporting electrolyte is in excess, or that the electric potential in the transport zone can be considered constant. Thus, the voltage drop (and the overvoltage) between the electrode and the bulk  solution can be assumed constant along the electrode surface. It makes the boundary conditions (7) linear relative to concentrations c1 and c2, and it allows to apply the common methods of mathematical physics. Such a solution, got by means of Fourie-transformation and expansion by Airy functions, is given in the Appendix B.

A few example of the concentration profiles c1-cO(1;0) in the horizontal sections of the cell are presented in Fig.6. Liquid motion in the cell moves counter clockwise. It can be shown, that as an elementary volume, taken beside the left electrode, moves down, it is being gradually saturated by the cations 1, and as it moves upwards along the other electrode, it loses its cation supply and gets to the upper part of the cell short of cations (see Fig.6). The curve for c2-cO(2;0) differs from Fig.6,7 only in the negative factor, that means, that the shortage of cations of kind 2 appears not in the upper, but in the lower part of the cell. Threedimensional surface, corresponding to the concentration distribution in the volume of TGC, is shown on the Fig.8.

The resulting current-voltage curve is approximately the same as for the case of diffusion-migration transport (Fig.3).

Conclusion.

In the present work the functioning of existing thermogalvanic cells has been theoretically studied. The analysis for the simplest case with account for only diffusion transport of the reacting substances from one electrode to another has been made, as well as for more complex cases: diffusion-migration transport for the horizontally arranged cell and diffusion-convection transport for the vertically arranged one. In all cases the volt-amper curves were calculated for TGC.

The qualitative analysis of the efficiency of TGC functioning has been performed. The brief algorithm for choosing the optimum parameters is the following:

1) The solvent with the lowest thermal conductivity should be chosen. Water solution can not be considered as a good one, although it is generally used.

2) The mixture of two substances, which can transform one into another by the

electrochemical reaction like (6), should be chosen. The maximum changes of

the equilibrium potential (6) with temperature should be possibly reached.

3) The temperature range for the working electrodes should be chosen, to provide for the optimum potential changes (see the previous item).

4) The concentrations of the components are to be high enough, but it should be taken into account, that the growth of the efficiency with the concentration is limited. Moreover, in the case of the horizontally arranged cell, when the migration transport is possible, too high concentration under certain conditions can lead to the following: one substance becomes a supporting electrolyte for another one, which can lead to reduction of the rate of matter transport and reduction of h .

5) The external resistance should be constant and have the same value, as TGC itself.

6)  Geometrical sizes of electrodes, the distance between them and their arrangement (horizontal or vertical), the temperature difference between the electrodes and the rate of liquid circulation inside the cell do not have a substantial influence on the results.

It is still a question of discussion, if the researchers succeed to find the optimum parameters of TGC by means of this algorithm. Now we have to admit, that the theoretical analysis of existing cells gives approximately the same evaluation for their efficiency h ~0.1%, as experimental measurements. Undoubtedly, it is not enough for any practical applications.

We think, that the further progress in this field will be connected with TGC of principally new type [15,16], in which the main volume between the electrodes is filled by gas phase.

Acknolegments:

This work was supported by Swedish Academy of Science. The author is grateful to Prof. F.Bark and Prof.Yu.I.Kharkats for many useful discussions.

Appendix A.

In this appendix the solution of the equation system (10)-(20) is presented and the parametric expression for the volt-amper dependence of TGC is obtained.

Let us introduce the dimensionless variables:

Zl =zl/z3 ;  Z2 =z2/z3 ;  Cm =cm/cO(3;0) ;  O(x;~) = F(x-xL;xR-xL) ;  k =F(z1cO(1;0) ;z3cO(3;0) )  ;

y1=y(O(x;~) =0); y2=y(O(x;~) =1);  O(I;~) = F(i(xR-xL);FD1cO(3;0)) ;  c = F(FD1cO(3;0);(xR-xL)i0) F(z1-z2;z1-z2D1/D2) ;

w1 = expB(F((z1-z2)xLF;2RTl)); w2 = expB(F((z1-z2)xRF;2RT2)); y = z3 F   (F;RT) j ;      (A1)

Then Nernst-Plank equations, condition of electroneutrality and boundary conditions may be presented as follows:

F(O(I;~);z1 -z2 ) =  F(dC1;dO(x;~)) + Z1 C1  F(dy;dO(x;~)) ;				(A2)

- F(O(I;~);z1 -z2 ) F(D1 ;D2 ) = F(dC2;dO(x;~)) + Z2 C2  F(dy;dO(x;~)) ;			(A3)

0 =     - F(dC3;dO(x;~)) + C3  F(dy;dO(x;~)) ,					(A4)

Z1 C1  + Z2 C2 = C3  					(A5)

I(0;1;C3dO(x;~)) =1; I(0;1;C2dO(x;~))=(1-k)/Z2 ,				(A6)

cJ= F(C1(x=0)Z1;kw1) -  F(C2(x=0)Z2w1;1-k);			(A7)

- cJ= F(C1(x=1)Z1;kw2) -  F(C2(x=1)Z2w2;1-k);			(A8)

where

J =  F(Z1-Z2D1/D2;Z1-Z2)O(I;~);					(A9)

Multiplying (A2) by Z1 , (A3) by Z2  and summing them up together with (A4), we get:

F(dO(x;~);dy) = B(F(dy;dO(x;~)))-1= B(Z2 (Z2 -Z1)C2 + (Z1 +1)C3 ) /J	(A10)

Expression (A10) allows us to transfer the independent variable O(x;~) in equations (A2)-(A4) for a new independent variable y . It immediately results from (A4), that C3 ~expy, and it results from (A3), that C2  is the linear combination of exponential functions of potential  expy  and exply, where l   is:

l  =  F(D1-D2;D2/z2-D1/z1)					(A11)

The unknown coefficients in the exponents are to be determined from the boundary conditions. It is convenient to represent the solution for C2, C3  in the form

C3  = Mey-y1

C2  = Mey-y1B(S + Ne(l-1)(y-y1))			(A12)

where the constant S  is calculated directly from the transformed equation (A3):

S = F((Z1 +1 )D1;(Z1D2 -Z2 D1 )(l -1)) .				(A13)

In order to determine constants M  and N, the integration variable in (A6) is changed and then (A10) is integrated, after that the obtained square equation should be solved relatively N :

N=N(w)= R(B(g2 -F(1-k;Z2 )g4)2(wl+1-1)2-4g1(w2l-1)B(g3-F(1-k;Z2)g5)( w2-1)) -

- B(g2 - F(1-k;Z2 )g4)(wl+1-1) F(1;2g1 (w2l-1)) ,

M=M(w)=  F(N (wl-1)g6 + (w-1)g7 ;N (wl+1-1)g4 + (w2-1)g5) ,

J=J(w)=  M [N (wl-1)g6 + (w-1)g7] .			(A14)

The designations are introduced for the potential drop in the transport region logw = y2 - y1  and for cumbersome algebraic combinations of charge numbers and diffusion coefficients:

g1 = Z2 (Z2 -Z1 )/2l ;

g2 = [2Z2 (Z2 -Z1 )S +Z1 +1]/(l + 1);

g3 = S [Z2 (Z2 -Z1 )S +Z1 +1]/2;

g4 = Z2 (Z2 -Z1 )/(l + 1);

g5 = [Z2 (Z2 -Z1 )S +Z1 +1]/2;

g6 = Z2 (Z2 -Z1 )/l ;									g7 = Z2 (Z2 -Z1 )S +Z1 +1.				(A15)

Comparison of the expressions (A14), (A12) and their combination with the definition (A1) result in the following:

C3(O(x;~)=0)= M;C3(O(x;~)=1)= Mw;C2(O(x;~)=0)= M(S+N);C2(O(x;~)=1)=M(Sw+Nwl)  (A16)

Values C1(O(x;~)=0)  and C1(O(x;~)=1) are derived from (A16) and (A5). Solving equations (A7), (A8) relatively w1, w2   we get:

w1=w1(w)= F(-cJ+ R((cJ)2+ F(4C1(O(x;~)=0)C2(O(x;~)=0)Z1Z2;k(1-k)));2C2(O(x;~)=0)/(1-k) )

w2=w2(w)= F(-cJ+ R((cJ)2+ F(4C2(O(x;~)=1)C2(O(x;~)=1)Z1Z2;k(1-k)));2C2(O(x;~)=1)/(1-k) )		 (A17)

The overvoltages valuesxL and xR  are determined from the correlation (A17) after replacing of dimensionless variables by dimensional variables:

xL = F(2RT1;F(z1-z2)) lnw1

xR = F(2RT2;F(z1-z2)) lnw2				 (A18)

As the dimensional potential drop in the transport zone is expressed just as F(RT;z3F) ew, equation (19) can be used for determination of the potential difference U=U(w) between the electrodes. Thus, the functions U(w) and J(w) are determined (see(A14)), and it means that the dependence I(U) is determined parametrically for all possible values of potential difference.

Appendix B.

In this appendix the solution of the convective diffusion problem for thermogalvanical cell with vertical arrangement of the electrodes will be given.

Introducing dimensionless concentrations  Cm=cm/c0 , it is possible to present equations (29) as

F(1;n) F(_2Cm;_n2 ) = qm F(_Cm;_r) , m = 1,2				(B1)

where

qm = F(qL2H2;2Dm) , m = 1,2 -					(B2)

is a dimensionless constant. Due to periodicity of Cm  in the segment (0,2), concentrations can be expanded into Fourie series by variable t; their components C(O(m;1)) fulfil the equations:

F(1;n) F(_2C(O(m;l)) ;_n2 ) = qm pilC(O(m;l))  , l_ 0				(B3)

F(_2C(O(m;0)) ;_n2 ) = 0, l= 0

Solutions of these equations, restricted at nÆ_ , are the following:

C(O(m;l)) = Ai(R(3;qmpil) n ), l _ 0					(B4)

C(O(m;0)) = CO(m;0),  l = 0

Airy function Ai tends to zero exponentially with the argument increase. The complementary Airy function of the second kind Bi increases unlimitedly and must be omitted from solution (B4). Airy function can be expressed by Bessel functions with index 1/3, and they are often applied in different problems of mathematical physics, especially in problems of quantum mechanics [17].

Summarizing the Fourie series

Cm = CO(m;0) + ISU(l=1;•;F(O(m;l))eilpt) AiB(R(3;qmpil) n ) 			(B5)

we get a solution, satisfying both periodical boundary conditions and initial conditions for the bulk volume far apart from the electrodes. The indefinite coefficients F(O(m;l))  should be determined from the boundary conditions (17), (18) for the electrode surfaces.

As the electrical current on the surface of the electrode is transported only by means of diffusion, we get I1 = z1 FD1 F(_c1;_x).  Taking into account stoichiometric relations for currents (14), (15), boundary conditions (17), (19) can be presented as:

- D1 F(_C1;_n)|n=0 = D2 F(_C2;_n)|n=0 = aC1|n=0 + bC2|n=0			(B6)

where a  and b  are simple step-like functions of coordinate t :

s w1/CO(1;0)    at 0<t  <1      	        -s /B(w1CO(2;0))    at 0<t  <1

a= 						b=				               		   (B7)

s w2/CO(1;0)    at 1<t  <2      	        -s /B(w2CO(2;0))    at 1<t  <2

and

s = F(i0H;C0(z1-z2)F) ; w1 = expB(F((z1-z2)xLF;2RTl)); w2 = expB(F((z1-z2)xRF;2RT2))    		(B8)

Introducing (B5) into the first equation (B6), multiplying it by e-ifpt, and integrating in the interval (0,2), we obtain a simple relation for the expansion coefficients.

F2(l)= - D2/D1 R(3;q2/q1)   F1(l)			  	 (B9)

and introducing (B6) into the second equation, we get

ISU(l=1;•;eilprB(m B(a F1(l)+b F2(l))+ m'R(3;q1pil) D1F1(l))) =-aC10 - bC20 	(B10)

Here m =Ai(0)= 0.3550 and m '=Ai'(0)=0.2588 are constant. Multiplying (B10) by e-ifpr, f=1,2,... and integrating in the interval (0,2) we get an infinite system of linear equations relatively F1l:

ISU	(l=1;•;GlfF1(l)) +  QfF1(f) = Vf				(B11)

where

F(2ims;2p(l-f)) BBC(B(F(w1-w2 ;c10 )) +F(D2 ;D1 ) R(3;F(q2;q1) )B(F(1;W1c20 ) - F(1;W2c20 ))) 	l-f- odd

Glf =      ms B(F(w1+w2 ;c10 ) + F(1;c20 ) B( F(1;w1) + F(1;w2)) )  				l=f

0 				in other cases	(B12)

Vf = 		F(2is;fp)   B(w1-w2- F(1;w1) - F(1;w2)) , f - odd			(B13)

0 			                f - even

qf=m' R(3;q1pif)D1					(B14)

The system of equations (B11)-(B14) is to be solved relatively F1l; and after that the solution of (B5) appears to be dependent only on two parameters, w1 and w2. They should be chosen with regard to a condition, that the integral current O(I;_), flowing through the surfaces of two electrodes, is defined by the external source:

I(0;l;I1(t)dt) = - I(l;2;I1(t)dt) = F(O(I;_),;1-z2/z1)				(B15)

So, we get two additional conditions for F1l :

ISU(f=1;_;l-2/3F1(l))= ± F(O(I;_),;1-z2/z1) F(C10ip;2m'D1FHz1 R(3;q1pi))			(B16)

Parameters w1  and w2  are to be found at fixedO(I;_) from equations (B16), and after that the value of voltage difference between the electrodes u will be determined from (B8) and (19). Practically, it is more convenient to use the parametric form of the volt-amper curve with intermediate parameter w1 and to define the value of the integral current from one of equations (B16). In this case we have to solve numerically only one equation relatively w2, because the solution of the system (B11) can be obtained analytically with any required accuracy.

There is also the theoretical evaluation of the number of those terms in the system (B11), which are to be taken into account. Actually, the solution in form (B5) is reduced to the expansion of the step-like boundary conditions (B6) by trigonometrical functions. Increasing of the terms number in (B5) leads to better approximation of the step-like function, but, as a matter of fact, the solution loses its accuracy in the upper and lower regions of the cell, i.e. in the area of L/H length around the points 0 and 1. It means, that taking into account the terms with their numbers, exceeding

Nmax= H/L,					(B17)

is out of the limits of accuracy. For typical systems H/L ~10, and the solution of system of equations (B11) does not involve any difficulties.

List of Symbols

a,b		function, corresponding to the boundary conditions

Ai		Airy function

O(c;^) 		specific thermal capacity, J/kg.K.

c0 		concentration of solution, mol/m3

ci		concentrations of cations species i, mol/m3

c3 		anions concentration, mol/m3

cO(i,0)		initial concentrations of ions species i, mol/m3

ci|S 	concentrations of ions species i at the electrode surface, mol/m3

Dc 		difference of the total concentration, mol/m3

Cm		dimensionless concentrations of ions species i  (i=1,2,3)

Dk,		diffusion coefficients of the k-th ion (k=1,2,3), m2/s

e		= 2.71828

O(e;Æ)y  		ort in the direction of axis y

f		index of adding

F		Faraday's constant, 96487 C/mol

Fm(l)		coefficients of Furie series for cations of kind m

g		gravitation constant, 9.81 m/s2

Glf		matrix of a linear system

H		cell height, m

i		density of electric current, A/m2

i		imaginary unity

i0		exchange current density, A/m2

i1 , i2  	current densities, transported by ion species 1 and 2, A/m2

I		electrical current, A

I0 		current of short circuit, A

O(I;~) 		dimensionless current density

O(I;_)		integral current, A

je 		maximum density of electric power, W/m2

jh 		thermal density flux, W/m2

J		renormed dimensionless current density

k		dimensionless initial concentration

k1, k2	rate constants of forward and back reactions, respectively

l		index of adding

L 		distance between electrodes, m

M 		constant of integration

Mz1, Mz2+.	ions of the redox couple

O(n;Æ)		axis, directed from the electrode into the solution

N 		constant of integration

p 		pressure, Pa

Qf		vector for linear system

r		inner resistance, W

r0 		inner resistance of the open circuit, W

R 		universal gas constant, 8.314 J/mol.K

Re		Reynolds number

Rext 		external resistance, W

s  		area of electrode, m2

S 		constant of integration

T 		absolute temperature, K

T0 		average values of temperature, K

Ti		temperature of electrodes (i=1,2), K

u0		potential drop between electrodes in zero approximation, V

uk  		mobility of the k-th ion (k=1,2,3), m2/s.V

U 		cell potential, V

U0, 		cell potential at open circuit, V

v		fluid velocity,  m/s

Vf		vector of the right part of linear system

w1 ,w2	intermediate parameter of theory

W		useful power, W

Wmax,	maximal useful power, W

O(x;~) 		dimensionless coordinate

x  		coordinate, directed from one electrode to anoter, m

xL, xR.	coordinates of electrodes, m

y 		coordinate, directed along the electrode surface, m

z1, z2	charge numbers of cations of the first and the second kinds, respectively

z3 		charge number of anions

Zl , Z2	ratios of charge numbers

a 		transfer coefficient

bC 	coefficient of dependence of the density on concentration, m3mol-1

bT  		thermal expansion coefficient, m3K-1

gi 		combinations of charge numbers and diffusion coefficients

h		 efficiency

hCarno	efficiency of the ideal thermodynamical system

q		parameter of theory, m-1

qm 		dimensionless parameter q,  m = 1,2

æh 		coefficient of thermal conductivity of liquid, W/m.K

æi 		specific resistance, W.m

æT		temperature conductivity coefficient, m2/s

l		dimensionless combination of diffusion coefficients

l0 		equivalent electroconductivity, W.m4/mol

m , m '	value of Airy function and its derivatives at zero

n  		kinetic viscosity, m2/s

x 		surface overpotential, V

r 		density of liquid, kg/m3

s		dimensionless exchange current density

O(t;Æ)		coordinate, directed along the surface of the electrodes

j 		potential, V

j (s)		equilibrium potential, V

F		stream function, m

c 	ratio of diffusion current density and exchange current density

y		dimensionless potential

y1, y2	values of a dimensionless potential at the electrodes

w		intermediate parameter

References

1	B.Burrows, Proc.Inters.Energy Convers.Eng.Conf.,10 (1975) 821.

2	B.Burrows, J.Electrochem.Soc., 123 (1976) 154.

3	H.L.Chum, Proc.Inters.Energy Convers.Eng.Conf.,15(1980) 1603.

4	T.I.Quickenden and C.F.Vernon, Sol.Energy,36 (1986) 63.

5	T.Ikeshoji, Bull.Chem.Soc.Jpn., 60 (1987) 1505.

6	T.Ikeshoji, J.Electroanal. Chem., 296 (1990) 19.

7	T.Ikeshoji, F.N.Nahui,  J.Electroanal. Chem., 305 (1991) 147.

8	T.Ikeshoji, F.N.Nahui, S.Kimura, M.Yoneya, J.Electroanal. Chem., 312 	(1992) 43.

9   Bagozkii V. Background  of Electrochemistry, Chimija, Moscow, (1988), (in Russian).

10 .K.Vetter. Electrochemische Kinetik. Springer-Verlag, Berlin, (1961).

11  Levich V.G. Physicochemical Hydrodynamics, Prentice-Hall, Inc., (1962).

12  Sokirko A.V.,Bark F., J.Electroanal. Chem, (in press).

13  J.Newman, Electrochemical systems.-2nd ed. Prentice-Hall (1991).

14  G.K.Batchelor, An introduction to fluid dynamics, Cambridge,(1988)

15  T.Ikeshoji, F.N.Nahui,  // 43rd Meet. of Int. Soc. of Electrochemistry - Cordoba, Argentina (1992) P.299.

16	Y. Maeda, E. Itoh, M. Inagaki, Synthic Metals, 20 (1987) 73.

17. Handbook of Mathematical Functions, M.Abramowitz and I.-N.Y 	Stegun(Eds.), National Bureau of Standards,Gaithersburg, MD,1964.

Captions to the figures

Fig.1. The principal electrical scheme of a TGC.

Fig.2. The typical dependences of the current I,  the effective voltage U,  the inner resistance r and the useful power from the external resistance Rext

Fig.3. The volt-amper curve  for systems Fe(CN)O(6 ;4+) | Fe(CN)O(6 ;3+) in the case of the diffusion- migration transport (stagnant electrolyte). Dots correspond to the uniform concentrations profiles.

Fig.4. The concentration profiles of ions, corresponding to optimum regime.

Fig.5. The scheme of a TGC for calculation of diffusion-convection transport.

Fig.6. The concentration of cations of the first kind along surfaces of the first electrode (0< t <1) and of the second one (1< t< 2).

Fig.7. The concentration profiles of cations of the first kind in the horizontal sections of TGC:

1 - upper part; 2 - medium part; 3 - lower part.

Fig.8. The concentration distribution of cations of the first kind as a function of coordinates x  and y  (see also Fig.6,7).

* Permanent address:A.N.Frumkin Institute of Electrochemistry, Russian Academy of Sciences, Leninsky Prospect 31, 117071 Moscow, Russia